==+== S&P 2025 Cycle 1 Review Form

==-== DO NOT CHANGE LINES THAT START WITH "==+==" OR "==*==".

==-== For further guidance, or to upload this file when you are done, go to:

==-== https://cycle1.sp2025.ieee-security.org/offline

==+== =====================================================================

==+== Begin Review #391

==+== Paper #391

==-== Title: Defense in System Management Mode: A Hardware-Level Approach

==-==        Against Crypto Ransomware

==+== Review Readiness

==-== Enter "Ready" if the review is ready for others to see:

Ready

==*== Paper Summary

==-==    1-2 paragraphs

This paper proposes a hardware-level crypto ransomware defense approach (RansomSMM). It leverages the NVMe controller to trigger a System Management Interrupt (SMI) whenever I/O is issued on the SSD. This switches the CPU into System Management Mode (SMM), and so the motherboard firmware takes control and performs ransomware detection based on contextual information from the previous CPU state. Ransomware detections are communicated with the NVMe controller, which decides how to handle the I/O request.

Usually, the detection solutions which are isolated from the OS have significant limitations. However, RansomSMM enables using information from the OS for ransomware detection while still being resilient to ransomware which gains the OS level control. Additionally, by notifying the NVMe controller of detection results, it remains compatable with existing SSD based backup techniques, allowing the detection to guide recovery. The paper provides some implementation and evaluation details and a security analysis.

==*== Technical Correctness

==-==    Categorize the technical correctness of the submission

==-== Choices:

==-==    1. No Apparent Flaws

==-==    2. Minor Issues

==-==    3. Fixable Major Issues

==-==    4. Substantial Major Issues

==-== Enter the number of your choice:

2. Minor Issues

==*== Technical Correctness Comments

It is unclear what the possible detection results are and what the responses are in each case. Understanding this is necessary to evaluate the effectiveness of the response. In particular, it is unclear whether the possible detection results are binary. Upon a non-positive detection result, will the entropy always be calculated? If not, there may be some unrepairable corruptions, since it may take a few apparently benign I/O requests before classifying a process as ransomware.

In Section 4, details on the specific detection mechanisms used in the implementation are missing.

Additionally, the sharing of a key between the SMM and NVMe controller mentioned in Section 3.5 is concerning. How is the authenticity of the symmetric key determined by the NVMe controller? Is there any spoofing attack where malware can act as the SMM in sending this key?

==*== Scientific Contribution

==-==    Check all that apply

==-== Choices:

==-==    1. Independent Confirmation of Important Results with Limited

==-==       Prior Research

==-==    2. Provides a New Data Set For Public Use

==-==    3. Creates a New Tool to Enable Future Science

==-==    4. Addresses a Long-Known Issue

==-==    5. Identifies an Impactful Vulnerability

==-==    6. Provides a Valuable Step Forward in an Established Field

==-==    7. Establishes a New Research Direction

==-==    8. Other

==-== Enter your choices, separated by commas:

6

==*== Scientific Contribution Comments

This paper more naturally integrates a defense against privlidged ransomware into systems without introducing any extra significant software or hardware layers (e.g. a hypervisor). This further improves efficiency over existing solutions by removing the overhead associated with extra components.

Sections 4.4 may be moved to section 3. It seems this concept would apply generally and is a more fundamental design component.

==*== Presentation

==-== Choices:

==-==    1. No Flaws in Presentation

==-==    2. Minor Flaws in Presentation

==-==    3. Major but Fixable Flaws in Presentation

==-==    4. Fails to communicate essential ideas

==-== Enter the number of your choice:

2

==*== Presentation Comments

Lacking technical details on the things mentioned above, which results in a incomplete understanding of how good state is maintained in the presence of ransomware.

==*== Comments to Authors

==-==    Strengths, weaknesses, philosophical differences, etc

This is a clear step forward towards providing a viable complete ransomware defense framework. Eliminating the need for a hypervisor by incorporating detection into the motherboard firmware is a unique contribution which more naturally integrates the defense.

The performance increase over RansomTag is significant and is a good step towards a more economial solution which enables a rich context for ransomware detection while defending against the system level ransomware.

There are some missing design details which make it difficult to evaluate the correctness in terms of a data restoration garuntee.

It is also unclear whether the key shared between the SMM and NVMe controller might be vulnerable to a spoofing attack.

==*== Recommended Decision

==-== Choices:

==-==    1. Accept with Meta Review

==-==    2. Accept with Noteworthy Concerns in Meta Review

==-==    3. Weak Reject (Can be Convinced by a Champion)

==-==    4. Reject

==-== Enter the number of your choice:

2

==*== Reviewer Confidence

==-== Choices:

==-==    1. Certain

==-==    2. Highly Confident

==-==    3. Fairly Confident

==-==    4. Little Confidence

==-== Enter the number of your choice:

3

==*== Comments for PC

==-== (hidden from authors)

==-==    Comments to the PC only, including limitations in confidence.

==*== Should this submission be reviewed by the Research Ethics Committee?

==-== Choices:

==-==    1. No

==-==    2. Yes

==-== Enter the number of your choice:

1

==*== Ethics Concerns

==-== (hidden from authors)

==-==    If you have answered the previous question with Yes, please

==-==    briefly explain why you think this paper should be reviewed by the

==-==    Research Ethics Committee (REC). This might be related to research

==-==    involving human subjects, vulnerability disclosure, large scale

==-==    data collection, user data processing, or other topics that may

==-==    raise an ethical consideration. The REC will check all flagged

==-==    papers.

==*== Noteworthy Concerns

==-== (hidden from authors)

==-==    Draft text of noteworthy concerns for the meta-review. This will

==-==    make the process later easier.

==+== Scratchpad (for unsaved private notes)

==+== End Review